SECTION 1 Identification

1.1. Product identifier

| Product form | : | Mixture |
| --- | --- | --- |
| Trade name | : | Vantablack 230 Paint |

1.2. Other means of identification

| Other means of identification | : | Solvent borne carbon-based paint |
| --- | --- | --- |

1.3. Recommended use of the chemical and restrictions on use

| Recommended use | : | To be applied as spray paint in a well-controlled environment under licence and guidance by the manufacturer |
| --- | --- | --- |

1.4. Supplier's details

| Manufacturer SurreyNanosystems Ltd East Side Business Park, Beach Road Newhaven, East Sussex, BN9 0FB United Kingdom T 01273 515899 technicalsupport@surreynanosystems.com |  |
| --- | --- |

1.5. Emergency phone number

| Emergency number | : | +44 (0) 3301336058 |
| --- | --- | --- |

SECTION 2 Hazard Identification

2.1. Classification of the substance or mixture

Classification according to Regulation (EC) No. 1272/2008 [CLP]

| Flammable liquids, Category 2 | H225 | Highly flammable liquid and vapor. |
| --- | --- | --- |
| Acute toxicity, oral, Category 5 | H303 | May be harmful if swallowed. |
| Skin sensitisation, Category 1B | H317 | May cause an allergic skin reaction. |
| Serious eye damage/eye irritation, Category 2A | H319 | Causes serious eye irritation. |
| Acute toxicity, Category 4 | H332 | Harmful if inhaled. |
| Specific target organ toxicity – Single exposure, Category 3, Narcosis | H336 | May cause drowsiness or dizziness. |
| Specific target organ toxicity – Repeated exposure, Category 2 | H373 | May cause damage to organs through prolonged or repeated exposure. |
| Long-term, chronic, aquatic hazard, Category 3 | H412 | Harmful to aquatic life with long lasting effects. |

Full text of H statements : see section 16

Adverse physicochemical, human health and environmental effects

Highly flammable liquid and vapour. May cause drowsiness or dizziness. Causes serious eye irritation. Harmful if inhaled.

2.2. Label elements

Labelling according to Regulation (EC) No. 1272/2008 [CLP]

| Hazard pictograms (CLP) | : |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GHS02 | GHS07 |  |  |  |  |
| Signal word (CLP) Contains | : : | Danger ACETONE |  |  |  |  |  |
| Hazard statements (CLP) | : | H225 – Highly flammable liquid and vapour H317 - May cause an allergic skin reaction. H332 – Harmful if inhaled H336 - May cause drowsiness or dizziness H412 – Harmful to aquatic life with long lasting effects. |  |  |  |  |  |
| Precautionary statements (CLP) EUH Statements | : : | P102 - Keep out of reach of children. P210 - Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P233 – Keep container tightly closed. P240 – Ground and bond container and receiving equipment. P241 – Use explosion proof electrical/ventilating/lighting equipment. P242 – Use only non-sparking tools. P243 – Take action to prvent static discharges. P261 – Avoid breathings vapours, spray. P271 - Use only outdoors or in a well-ventilated area. P273 – Avoid release to the environment. P280 – Wear eye protection, protective gloves. P303+P361+P353 - IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water or shower. P304 + P340 - IF INHALED: Remove person to fresh air and keep comfortable for breathing. P305 + P351+P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P312 - Call a POISON CENTER or doctor/physician if you feel unwell. P337 + P313 - If eye irritation persists: Get medical advice/attention. P370 + P378 - In case of fire: Use CO2, dry chemical, or foam to extinguish. P403 + P233 - Store in a well-ventilated place. Keep container tightly closed. P403 + P235 - Store in a well-ventilated place. Keep cool. P405 - Store locked up. P501 - Dispose of contents and container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation. EUH066 - Repeated exposure may cause skin dryness or cracking. EUH208 - Contains 2-Hydroxyethyl methacrylate and Methyl methacrylate. May produce an allergic reaction. |  |  |  |  |  |

2.3. Other Hazards

Contains no PBT and/or vPvB substances ≥ 0.1% assessed in accordance with REACH Annex XIII

The mixture does not contain substance(s) included in the list established in accordance with Article 59(1) of REACH for having endocrine disrupting properties, or substance(s) are not identified as having endocrine disrupting properties in accordance with the criteria set out in Commission Delegated Regulation (EU) 2017/2100 or Commission Regulation (EU) 2018/605 at a concentration equal to or greater than 0,1 %

SECTION 3 Composition/information on ingredients

3.1. Substances

Not applicable

3.2. Mixtures

| Name | Product identifier | % | Classification according Regulation (EC) No. 1272 [CLP] |
| --- | --- | --- | --- |
| Binder | CAS-No.: N/A | 5 – 25 | - |
| Acetone | CAS-No.: 67-64-1 EC-No.: 200-662-2 EC Index-No.: 606-001-00-8 | 20 – 38 | Flam. Liq. 2, H225 Eye Irrit. 2, H319 STOT SE 3, H336 |
| Ethyl acetate | CAS-No.: 141-78-6 EC-No.: 205-500-4 EC Index-No.: 607-022-00-5 | 5 – 25 | Flam. Liq. 2, H225 Eye Irrit. 2, H319 STOT SE 3, H336 |
| Toluene | CAS-No.: 108-88-3 EC-No.: 203-625-9 EC Index-No.: 601-021-00-3 | 0.1 – 0.7 | Flam. Liq. 2, H225 Skin Irrit. 2, H315 Repr. 2, H361 STOT SE 3, H336 STOT RE 2, H373 Asp. Tox. 1, H304 Aquatic Acute 2, H401 Aquatic Chronic 3, H412 |
| Trimethoxyvinylsilane | CAS-No.: 2768-02-7 EC-No.: 220-449-8 EC Index-No.: 014-049-00-0 | 0.35 – 3.5 | Flam. Liq. 3, H226 Acute Tox. 4 (Inhalation), H332 Skin Sens. 1B, H317 |

Full text of H- and EUH-phrases: see SECTION 16.

This mixture does not contain further substances fulfilling the criteria of hazard class "acute toxicity" according to CLP regulation.

This product does not contain substances of very high concern (Regulation (EC) No 1907/2006 (REACH), Article 57) in amounts above ≥0.1%.

SECTION 4 First aid measures

4.1. Description of necessary first-aid measures

| First-aid measures general | : | IF exposed or concerned: Get medical advice/attention. First aider: Pay attention to self-protection. Never give anything by mouth to an unconscious person. Give artificial respiration if necessary. Induce artificial respiration with mask fitted with one-way valve or other suitable device but, not mouth-to-mouth. |  |
| --- | --- | --- | --- |
| First-aid measures after inhalation | : | If breathing is difficult, remove victim to fresh air and keep at rest in a position comfortable for breathing. If the victim is unconscious: Lay in a stable manner on victim's side. Induce artificial respiration with mask fitted with one-way valve or other suitable device; not mouth-to-mouth. Call a physician immediately. |  |
| First-aid measures after skin contact | : | Remove affected clothing and wash all exposed skin areas with mild soap and water, followed by warm water rinse. If skin irritation or rash occurs: Get medical advice/attention. Wash contaminated clothing before reuse. |  |
| First-aid measures after eye contact | : | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If eye irritation persists: Get medical advice/attention. |  |
| First-aid measures after ingestion | : | Rinse mouth and spit the fluids out. Do NOT induce vomiting. If vomiting occurs, the head should be kept low so that vomit does not enter the lungs. Call a poison center/doctor/physician if you feel unwell. |  |
| Self protection of the first-aider | : | First aid workers will be equipped with suitable personal protective equipment. |  |

4.2. Most important symptoms/effects, acute and delayed

| Symptoms/effects after inhalation | : | Depression of the central nervous system, headaches, dizziness, drowsiness, loss of coordination. |
| --- | --- | --- |
| Symptoms/effects after skin contact | : | May cause an allergic skin reaction. Repeated exposure may cause skin dryness or cracking. |
| Symptoms/effects after eye contact | : | Stinging, redness, itching, tears, blurred vision, swelling. |
| Symptoms/effects after ingestion | : | Ingestion may cause nausea and vomiting. Gastrointestinal disturbances. |
| Chronic Symptoms | : | Suspected of damaging the unborn child. |

4.3. Indication of immediate medical attention and special treatment needed, if necessary

| Other medical advice or treatment | : | Treat symptomatically. IF exposed or concerned: Get medical advice/attention |
| --- | --- | --- |

SECTION 5: Fire-fighting measures

5.1. Suitable (and unsuitable) extinguishing media

| Suitable extinguishing media | : | Alcohol-resistant foam. Carbon dioxide. |
| --- | --- | --- |
| Unsuitable extinguishing media | : | Do not use water |

5.2. Specific hazards arising from the chemical

| Fire hazard | : | Highly flammable liquid and vapour. |
| --- | --- | --- |
| Explosion hazard | : | Vapours are heavier than air and may spread near the ground, travel a considerable distance to a source of ignition and flash back. Severe explosion hazard when vapours ignite. |
| Hazardous decomposition products in case of fire | : | Toxic fumes may be released. |

5.3. Special protective equipment and precautions for fire-fighters

| Firefighting instructions | : | Fight fire from safe distance and protected location. Do not enter fire area without proper protective equipment, including respiratory protection. |
| --- | --- | --- |
| Protection during firefighting | : | Do not attempt to take action without suitable protective equipment. Self-contained breathing apparatus. Complete protective clothing. |

SECTION 6 Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

| General measures | : | Avoid all personal contact including breathing in the mist, spray, vapors. Do not take actions involving personal risks. Absorb spillage to prevent material-damage. Stop leak if safe to do so. Notify authorities if product enters sewers or public waters. |
| --- | --- | --- |

For non-emergency personnel

| Protective equipment | : | Wear recommended personal protective equipment. |
| --- | --- | --- |
| Emergency procedures | : | Ventilate spillage area. No open flames, no sparks, and no smoking. Avoid breathing dust/fume/gas/mist/vapours/spray. Avoid contact with skin and eyes. |

For emergency responders

| Protective equipment | : | Wear the recommended personal protective equipment. Do not attempt to take action without suitable protective equipment. For further information refer to section 8: "Exposure controls/personal protection". |  |
| --- | --- | --- | --- |
| Emergency procedures | : | Evacuate unnecessary personnel. Ventilate spillage area. Stop leak if safe to do so. Prevent from entering sewers, basements and workpits, or any place where its accumulation can be dangerous. All equipment used when handling the product must be grounded. |  |
| Environmental precautions | : | Do not let the product reach soil, drains, sewers, or surface and ground water. Notify authorities if product enters sewers or public waters. |  |

6.2. Environmental Precautions

Avoid Release to the environment

6.3. Methods and materials for containment and cleaning up

| For containment | : | Absorb spilled material with sand or earth. Contain any spills with dikes or absorbents to prevent migration and entry into sewers or streams. Stop leak without risks if possible. |
| --- | --- | --- |
| Methods for cleaning up | : | Isolate all ignition sources. Stop leak if safe to do so. Contain and collect spillage with non-combustible, absorbent material e.g. sand, earth, vermiculite or diatomaceous earth and place in container for disposal according to local regulations (see Section 13). Preferably clean with a water-based detergent. Avoid using solvents. |
| Other Information. | : | Dispose of materials or solid residues at an authorized site. |
| For further information refer to section 8: "Exposure controls/personal protection",For further information refer to section 13 |  |  |

SECTION 7 Handling and storage

7.1. Precautions for safe handling

| Precautions for safe handling | : | Prevent the creation of flammable or explosive concentrations of vapours in air and avoid vapour concentrations higher than the occupational exposure limits. In addition, the product should only be used in areas from which all naked lights and other sources of ignition have been excluded. Electrical equipment should be protected to the appropriate standard. Take precautionary measures against electrostatic discharge. Operators should wear antistatic footwear and clothing and floors should be of the conducting type. Keep away from heat, sparks, and flame. Non-sparking tools should be used. Avoid contact with skin and eyes. Avoid the inhalation of dust, particulates, spray or mist arising from the application of this mixture. Eating, drinking and smoking should be prohibited in areas where this material is handled, stored and processed. Keep container tightly closed and upright when not in use. Vapours are heavier than air and may spread along floors. Vapours may form explosive mixtures with air. |
| --- | --- | --- |
| Hygiene measures | : | Do not eat, drink or smoke when using this product. Always wash hands after handling the product. |
| Additional hazards when processed | : | Not expected to present a significant hazard under anticipated conditions of normal use. |

7.2. Conditions for safe storage, including incompatibilities

| Storage conditions | : | Store in a cool, dry and well-ventilated area away from incompatible substances. Store containers in location free from fire risk and away from sources of heat and ignition. Protect from sunlight. Always keep container in upright position. Store between 0 and 25°C. Store in accordance with local regulations. Containers that have been opened must be carefully resealed and kept upright to prevent leakage. |
| --- | --- | --- |
| Incompatible materials | : | Strong acids. Strong bases. Strong oxidizing agents. |

7.3 Specifc end use(s)

| To be used as a spray paint coating for engineering applications |  |
| --- | --- |

SECTION 8 Exposure controls/personal protection

8.1. Control parameters

| ACETONE (67-64-1) |  |
| --- | --- |
| Occupational Exposure Limits |  |
| Local name | Acetone |
| WEL TWA (OEL TWA) IOELV TWA (OEL TWA) | 1210 mg/m³ |
|  | 550 ppm |
| WEL STEL (OEL STEL) IOELV TWA (OEL STEL) | 3620 mg/m³ |
|  | 1500 ppm |
| Regulatory reference | EH40/2005 (Fourth edition, 2020). HSE, Directive 2009/161/EU |

| ETHYL ACETATE (141-78-6) |  |
| --- | --- |
| Occupational Exposure Limits |  |
| Local name | Acetone |
| WEL TWA (OEL TWA) IOELV TWA (OEL TWA) | 734 mg/m³ |
| OSHA PEL TWA | 200 ppm |
| WEL STEL (OEL STEL) IOELV TWA (OEL STEL) | 1468 mg/m³ |
|  | 400 ppm |
| Regulatory reference | EH40/2005 (Fourth edition, 2020). HSE, Directive 2009/161/EU |

| Toluene (108-88-3) |  |
| --- | --- |
| Occupational Exposure Limits |  |
| Local name | Toluene |
| WEL TWA (OEL TWA) IOELV TWA (OEL TWA) | 192 mg/m³ |
|  | 50 ppm |
| WEL STEL (OEL STEL) IOELV TWA (OEL STEL) | 384 mg/m³ |
|  | 100 ppm |
| Regulatory reference | EH40/2005 (Fourth edition, 2020). HSE, Directive 2009/161/EU |

8.2. Appropriate engineering controls

| Appropriate engineering controls | : | Ensure good ventilation of the work station. Use general ventilation, local exhaust ventilation, or process enclosure to keep the airborne concentrations below the permissible exposure limits. Emergency eye wash fountains and safety showers should be available in the immediate vicinity of any potential exposure. |  |
| --- | --- | --- | --- |
| Environmental exposure controls | : | Avoid release to the environment. Take measures to reduce or limit air emissions and releases to soil and the aquatic environment. |  |

8.3. Individual protection measures, such as personal protective equipment

Personal protective equipment:

Wear recommended personal protective equipment. Avoid all unnecessary exposure.

Hand protection:

For prolonged or repeated contact use protective gloves. Barrier creams may help to protect the exposed areas of skin, they should however not be applied once exposure has occurred. Skin should be washed after contact. Use chemical resistant gloves classified under Standard EN 374: Protective gloves against chemicals and micro-organisms. Recommended gloves: Natural rubber, Neoprene, Barrier glove.

Eye protection:

Wear safety glasses which protect from splashes

Skin and body protection:

Personnel should wear antistatic clothing. This should be made of natural fibres or of high temperature-resistant synthetic fibres. Appropriate footwear and any additional skin protection measures should be selected based on the task being performed and the risks involved and should be approved by a specialist before handling this product.

Respiratory protection:

In case of insufficient ventilation, wear suitable respiratory equipment

Personal protective equipment symbol(s):

Other information:

Do not eat, drink or smoke during use.

SECTION 9 Physical and chemical properties

9.1. Basic physical and chemical properties

| Physical state | : | Liquid |
| --- | --- | --- |
| Appearance | : | High Viscosity Liquid |
| Color | : | Black |
| Odor | : | Sweet |
| Odor threshold | : | 19.8 ppm |
| pH | : | No data available |
| Melting point | : | Not applicable |
| Freezing point | : | No data available |
| Boiling point | : | 56°C at 1.013 hPa (Acetone) |
| Flash point | : | - 16 °C Abel method Closed cup (ISO13736) |
| Flammability (solid, gas) | : | Highly flammable liquid and vapours |
| Vapor pressure | : | 24 kPa at 20°C (Acetone) |
| Relative vapor density at 20°C | : | 0.97 mg/ml |
| Relative density | : | No data available |
| Solubility | : | No data available |
| Partition coefficient n-octanol/water (Log Pow) | : | No data available |
| Auto-ignition temperature | : | 465 °C / 869 °F |
| Decomposition temperature | : | No data available |
| Viscosity, kinematic | : | ~3500 cP |
| Explosion limits | : | Lower explosion limit: 2.2 vol % Upper explosion limit: 13 vol % |
| Particle characteristics | : | No data available |

9.2. Other Information

| No additional information available. |  |  |
| --- | --- | --- |

SECTION 10 Stability and reactivity

10.1. Reactivity

Highly flammable liquid and vapour.

10.2. Chemical stability

Stable under normal conditions of use.

10.3. Possibility of hazardous reactions

No dangerous reactions known under normal conditions of use.

10.4. Conditions to avoid

Avoid contact with hot surfaces, heat, flames, and sparks. Eliminate all sources of ignition.

10.5. Incompatible materials

Strong acids. Strong bases. Strong oxidizing agents.

10.6. Hazardous decomposition products

Under normal conditions of storage and use, hazardous decomposition products should not be produced. Thermal decomposition generates: Carbon monoxide. Carbon dioxide.

SECTION 11 Toxicological information

11.1. Information on toxicological effects

| Acute toxicity (oral) | : | May be harmful if swallowed |
| --- | --- | --- |
| Acute toxicity (dermal) | : | Not classified |
| Acute toxicity (inhalation) | : | Harmful if inhaled |

| Binder |  |
| --- | --- |
| LD50 oral rat | > 2000 mg/kg body weight |

| Acetone (67-64-1) |  |
| --- | --- |
| LD50 oral rat | 5800 mg/kg body weight |
| LD50 dermal rabbit | > 15800 mg/kg |
| LC50 Inhalation - Rat | > 20 mg/l/4h |
| LC50 Inhalation - Rat (Vapors) | 76 mg/l |

| Ethyl acetate (141-78-6) |  |
| --- | --- |
| LD50 oral rat | 11.3 ml/kg |
| LD50 oral | 4934 mg/kg body weight |
| LD50 dermal rabbit | > 20000 mg/kg body weight |
| LD50 dermal | 18000 mg/kg |
| LC50 Inhalation - Rat | 4000 mg/l |
| LC50 Inhalation - Rat (Vapors) | 52.75 mg/l/4h |

| Stabilisers |  |
| --- | --- |
| LD50 oral rat | > 7000 mg/kg body weight |
| LD50 dermal rat | > 3000 mg/kg body weight |

| Toluene |  |
| --- | --- |
| LD50 oral rat | 5580 mg/kg body weight |
| LD50 dermal rabbit | > 5000 mg/kg body weight |
| LC50 Inhalation - Rat | 12.5 mg/l |

| Skin corrosion/irritation | : | Not classified |
| --- | --- | --- |

| Acetone |  |
| --- | --- |
| pH | 5 |

| Ethyl acetate |  |
| --- | --- |
| Skin corrosion/irritation, rabbit | Negative, Mildly irritating |

| Serious eye damage/irritation | : | Causes serious eye irritation. |
| --- | --- | --- |

| Acetone |  |
| --- | --- |
| pH | 5 |

| Ethyl acetate |  |
| --- | --- |
| Serious eye damage/irritation, rabbit | Severely irritating to the eyes |

| Respiratory or skin sensitization | : | May cause an allergic skin reaction. |
| --- | --- | --- |

| Ethyl acetate |  |
| --- | --- |
| Guinea pig maximization test | Not sensitive |

| Germ cell mutagenicity | : | Not classified |
| --- | --- | --- |

| Carcinogenicity | : | Not classified |
| --- | --- | --- |

| Toluene |  |
| --- | --- |
| IARC group | 3 - Not classifiable |

| Reproductive toxicity | : | Suspected of damaging the unborn child. |
| --- | --- | --- |

| Acetone |  |
| --- | --- |
| LOAEL (animal/female, F0/P) | 11298 mg/kg body weight |
| NOAEL (animal/male, F0/P) | 900 mg/kg body weight |

| Toluene |  |
| --- | --- |
| LOAEL (animal/female, F0/P) | 520 mg/kg body weight |

| STOT-single exposure | : | May cause drowsiness or dizziness. |
| --- | --- | --- |

| Binder |  |
| --- | --- |
| STOT-single exposure | May cause drowsiness or dizziness. |

| Acetone |  |
| --- | --- |
| STOT-single exposure | May cause drowsiness or dizziness. |

| Ethyl acetate |  |
| --- | --- |
| STOT-single exposure | May cause drowsiness or dizziness. |

| Toluene |  |
| --- | --- |
| STOT-single exposure | May cause drowsiness or dizziness. |

| STOT-repeated exposure | : | Not classified |
| --- | --- | --- |

| Acetone |  |
| --- | --- |
| NOAEL (oral,rat,90 days) | 900 mg/kg bw/day |

| Ethyl acetate |  |
| --- | --- |
| LOAEL (oral,rat,90 days) | 3600 mg/kg body weight |
| NOAEL (oral,rat,90 days) | 900 mg/kg body weight |

| Toluene |  |
| --- | --- |
| LOAEL (oral,rat,90 days) | 1250 mg/kg body weight |
| NOAEL (oral,rat,90 days) | 625 mg/kg body weight |
| NOAEC (inhalation,rat,vapor,90 days) | 2.355 mg/l air |
| STOT-repeated exposure | May cause damage to organs (central nervous system) through prolonged or repeated exposure (Inhalation). |

| Aspiration hazard | : | Not classified |
| --- | --- | --- |
| Symptoms/effects after inhalation | : | Depression of the central nervous system, headaches, dizziness, drowsiness, loss of coordination. |
| Symptoms/effects after skin contact | : | May cause an allergic skin reaction. Repeated exposure may cause skin dryness or cracking. |
| Symptoms/effects after eye contact | : | Eye irritation. Redness, itching, tears. |
| Symptoms/effects after ingestion | : | May cause gastrointestinal irritation, nausea, vomiting and diarrhea. |
| Chronic symptoms | : | Suspected of damaging the unborn child. |

SECTION 12 Ecological information

12.1. Ecotoxicity

| Hazardous to the aquatic environment, short–term (acute) | : | Not classified |
| --- | --- | --- |
| Hazardous to the aquatic environment, long–term (chronic) | : | Not classified |

| Acetone |  |
| --- | --- |
| LC50 - Fish [1] | 8300 mg/l |
| EC50 - Crustacea [1] | 8450 mg/l |
| ErC50 algae | 7200 mg/l |
| LOEC (chronic) | > 79 mg/l |
| NOEC (chronic) | ≥ 79 mg/l |
| NOEC chronic crustacea | 2212 mg/l |

| Ethyl acetate |  |
| --- | --- |
| LC50 - Fish [1] | 230 mg/l |
| EC50 - Crustacea [1] | 262 mg/l |
| NOEC (chronic) | 2.4 mg/l |
| NOEC chronic crustacea | 2.4 mg/l |

| Toluene |  |
| --- | --- |
| LC50 - Fish [1] | 5.5 mg/l |
| LOEC (chronic) | 2.76 mg/l |
| NOEC chronic fish | 1.39 mg/l |
| NOEC chronic crustacea | 0.74 mg/l |

12.2. Persistence and degradability

| Vantablack 310 Paint Aerosol |  |
| --- | --- |
| Persistence and degradability | Not established. |

| Binder |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |

| Acetone |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |

| Ethyl acetate |  |
| --- | --- |
| Persistence and degradability | Readily biodegradable. |

| Stabilisers |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |

| Petroleum gases, liquefied |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Toluene |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

12.3. Bioaccumulative potential

| Vantablack 310 Paint Aerosol |  |
| --- | --- |
| Bioaccumulative potential | Not established. |

| Acetone |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | -0.24 |

| Ethyl acetate |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 0.73 |
| Bioaccumulative potential | Bioaccumulation unlikely. |

| Petroleum gases, liquefied |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | ≤ 2.8 |

12.4. Mobility in soil

No additional information available

12.5. Results of PBT and vPvB assessment

| No additional information available |  |  |
| --- | --- | --- |

## 12.6. Endocrine disrupting properties

| No additional information available |  |  |
| --- | --- | --- |

## 12.7. Other adverse effects

| No additional information available |  |  |
| --- | --- | --- |

SECTION 13 Disposal considerations

| Regional waste regulation | : | Disposal must be done according to official regulations. |
| --- | --- | --- |
| Waste treatment methods | : | Dispose of contents/container in accordance with licensed collector’s sorting instructions. |
| Sewage disposal recommendations | : | Disposal must be done according to official regulations. |
| Product/Packaging disposal recommendations | : | Disposal must be done according to official regulations. |
| Additional information | : | Do not re-use empty containers. |
| Ecological waste information | : | Avoid release to the environment. |

SECTION 14 Transport information

In accordance with ADR / ADN / IMDG / IATA

|  | Land Transport (ADR/RID) | Inland waterway transport (ADN) | Sea Transport (IMDG) | Air transport (ICAO-TI/IATA-DGR) |
| --- | --- | --- | --- | --- |
| 14.1. UN number | 1263 | 1263 | 1263 | 1263 |
| 14.2. Proper Shipping Name | Paint | Paint | Paint | Paint |
| 14.3. Transport hazard class(es) | 3 | 3 | 3 | 3 |
| Hazard Labels | Flam. liq. | Flam. liq. | Flam. liq. | Flam. liq. |
| 14.4. Packing group | 2 | 2 | 2 | 2 |
| 14.5. Environmental hazards | unknown | unknown | unknown | unknown |

14.6. Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code

Not applicable

14.7. Special precautions for user

Transport within user’s premises: Always transport in closed containers that are upright and secure. Ensure that persons transporting the product know what to do in the event of an accident or spillage.

SECTION 15 Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

| EU Regulations |  |  |
| --- | --- | --- |
| REACH Annex XVII (Restriction List) | : | Contains no substance(s) listed on REACH Annex XVII (Restriction Conditions) |
| REACH Annex XIV (Authorisation List) | : | Contains no substance(s) listed on REACH Annex XIV (Authorisation List) |
| REACH Candidate List (SVHC) | : | Contains no substance(s) listed on the REACH Candidate List |
| PIC Regulation (Prior Informed Consent) | : | Contains no substance(s) listed on the PIC list (Regulation EU 649/2012 concerning the export and import of hazardous chemicals) |
| POP Regulation (Persistent Organic Pollutants) | : | Contains no substance(s) listed on the POP list (Regulation EU 2019/1021 on persistent organic pollutants) |
| Ozone Regulation (2024/590) | : | Contains no substance(s) listed on the Ozone Depletion list (Regulation EU 2024/590 on substances that deplete the ozone layer) |
| Council Regulation (EC) for the control of dual-use items | : | Contains no substance subject to the COUNCIL REGULATION (EC) for the control of dual-use items |

Explosives Precursors Regulation (EU 2019/1148) Does not contain substance(s) listed on the Explosives Precursors list (Regulation EU 2019/1148 on the marketing and use of explosives precursors) ANNEX II REPORTABLE EXPLOSIVES PRECURSORS List of substances on their own or in mixtures or in substances for which suspicious transactions and significant disappearances and thefts are to be reported within 24 hours.

Drug Precursors Regulation (EC 273/2004) Does not contain substance(s) listed on the Drug Precursors list (Regulation EC 273/2004 on the manufacture and the placing on market of certain substances used in the illicit manufacture of narcotic drugs and psychotropic substances)

15.2. EU-Regulations

EU-Regulations

EU regulations: Not a hazardous substance according to the summation method of Regulation (EC) 1272/2008 (CLP), its various amendments and adaptations and Directive 67/548/EEC.

Authorisations and/or restrictions on use

Authorisations : Not applicable

Restrictions on use : Not applicable

Other EU regulations:

None

VOC content 485g/L in compliance with EU Directive 2004/42/CE Special Finish coatings.

15.3. Chemical Safety Assessment

For this mixture a chemical safety assessment is not required.

SECTION 16 Other information

| 16.1. Full text of hazard classes and H-statements |  |
| --- | --- |
| H225 | Highly flammable liquid and vapor |
| H303 | May be harmful if swallowed. |
| H304 | May be fatal if swallowed and enters airways |
| H315 | Causes skin irritation |
| H317 | May cause an allergic skin reaction |
| H319 | Causes serious eye irritation |
| H332 | Harmful if inhaled |
| H336 | May cause drowsiness or dizziness |
| H361 | Suspected of damaging fertility or the unborn child |
| H373 | May cause damage to organs through prolonged or repeated exposure |
| H401 | Toxic to aquatic life |
| H402 | Harmful to aquatic life |
| H412 | Harmful to aquatic life with long lasting effects |
| EUH066 | Repeated exposure may cause skin dryness or cracking. |
| EUH208 | Contains 2-Hydroxyethyl methacrylate and Methyl methacrylate. May produce an allergic reaction. |

| 16.2. Full text of precautionary statements and P-statements |  |
| --- | --- |
| P210 | Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. |
| P233 | Keep container tightly closed |
| P240 | Ground and bond container and receiving equipment |
| P241 | Use explosion-proof electrical/ventilating/lighting/equipment |
| P242 | Use only non-sparking tools. |
| P243 | Take action to prevent static discharges. |
| P261 | Avoid breathing vapours, spray. |
| P271 | Use only outdoors or in a well-ventilated area. |
| P280 | Wear eye protection, protective gloves. |
| P303+P361+P353 | IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water or shower. |
| P304 + P340 | IF INHALED: Remove person to fresh air and keep comfortable for breathing. |
| P305 + P351+P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P312 | Call a POISON CENTER or doctor/physician if you feel unwell. |
| P337 + P313 | If eye irritation persists: Get medical advice/attention. |
| P370 + P378 | In case of fire: Use CO2, dry chemical, or foam to extinguish |
| P403 + P233 | Store in a well-ventilated place. Keep container tightly closed |
| P403 + P235 | Store in a well-ventilated place. Keep cool |
| P405 | Store locked up. |
| P501 | Dispose of contents and container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation |

| 16.3. Abbreviations and acronyms |  |
| --- | --- |
| ACGIH | American Conference of Government Industrial Hygienists |
| ADN | European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways |
| ADR | European Agreement concerning the International Carriage of Dangerous Goods by Road |
| ATE | Acute Toxicity Estimate |
| BCF | Bioconcentration factor |
| BLV | Biological limit value |
| BOD | Biochemical oxygen demand (BOD) |
| CAS-No. | Chemical Abstract Service number |
| CLP | Classification Labelling Packaging Regulation; Regulation (EC) No 1272/2008 |
| COD | Chemical oxygen demand (COD) |
| CSA | Chemical safety assessment |
| DMEL | Derived Minimal Effect level |
| DNEL | Derived-No Effect Level |
| EC-No. | European Community number |
| EC50 | Median effective concentration |
| ED | Endocrine disruptor |
| EN | European Standard |
| EWC | European waste catalogue |
| IARC | International Agency for Research on Cancer |
| IATA | International Air Transport Association |
| IMDG | International Maritime Dangerous Goods |
| LC50 | Median lethal concentration |
| LD50 | Median lethal dose |
| LOAEL | Lowest Observed Adverse Effect Level |
| Log Kow | Partition coefficient n-octanol/water (Log Kow) |
| Log Pow | Partition coefficient n-octanol/water (Log Pow) |
| MAK | maximum workplace concentration |
| NOAEC | No-Observed Adverse Effect Concentration |
| NOAEL | No-Observed Adverse Effect Level |
| NOEC | No-Observed Effect Concentration |
| N.O.S. | Not Otherwise Specified |
| OECD | Organization for Economic Co-operation and Development |
| OEL | Occupational Exposure Limit |
| OSHA | Occupational Safety Health Administration |
| PBT | Persistent Bioaccumulative Toxic |
| PNEC | Predicted No-Effect Concentration |
| PPE | Personal protection equipment |
| RID | Regulations concerning the International Carriage of Dangerous Goods by Rail |
| SDS | Safety Data Sheet |
| STP | Sewage treatment plant |
| TF | Technical function |
| ThOD | Theoretical oxygen demand (ThOD) |
| TLM | Median Tolerance Limit |
| TWA | Time Weighted Average |
| VOC | Volatile Organic Compounds |
| vPvB | Very Persistent and Very Bioaccumulative |
| UFI | Unique Formula Identifier |

## 16.3. Training Advice

Hands on training to be administered by authorised representatives of Surrey Nanosystems Ltd only. Use of the material without training is done so at operator’s risk.

## 16.4. Further Information

Further information: IMPORTANT NOTE The information in this data sheet is not intended to be exhaustive and is based on the present state of our knowledge and on current laws: any person using the product for any purpose other than that specifically recommended in the technical data sheet without first obtaining written confirmation from us as to the suitability of the product for the intended purpose does so at his own risk. It is always the responsibility of the user to take all necessary steps to fulfil the demands set out in the local rules and legislation. Always read the Material Data Sheet and the Technical Data Sheet for this product if available. All advice we give or any statement made about the product by us (whether in this data sheet or otherwise) is correct to the best of our knowledge but we have no control over the quality or the condition of the substrate or the many factors affecting the use and application of the product. Therefore, unless we specifically agree in writing otherwise, we do not accept any liability whatsoever for the performance of the product or for any loss or damage arising out of the use of the product. All products supplied and technical advice given are subject to our standard terms and conditions of sale. You should request a copy of this document and review it carefully. The information contained in this data sheet is subject to modification from time to time in the light of experience and our policy of continuous development. It is the user's responsibility to verify that this data sheet is current prior to using the product. Brand names mentioned in this data sheet are trademarks of or are licensed to Surrey Nanosystems Ltd