SECTION 1 Identification

1.1. Product identifier

| Product Form | : | Mixture |
| --- | --- | --- |
| Trade name | : | Vantablack 230 Catalyst |

1.2. Other means of identification

| Other means of identification CAS No. EC No. REACH No. | : : : : | Metal Alkoxide Catalyst 5593-70-4 227-006-8 This product is a mixture and therefore not directly subject of the registration requirements under REACH. |
| --- | --- | --- |

1.3. Recommended use of the chemical and restrictions on use

| Recommended use | : | Catalyst has to be mixed with Vantablack 230 paint for application as a spray paint in a well-controlled environment under licence and guidance by the manufacturer. |
| --- | --- | --- |

1.4. Supplier's details

| Supplier SurreyNanosystems Ltd East Side Business Park, Beach Road Newhaven, East Sussex, BN9 0FB United Kingdom T 01273 515899 technicalsupport@surreynanosystems.com |  |
| --- | --- |

1.5. Emergency phone number

| Emergency number | : | +44 (0) 3301336058 |
| --- | --- | --- |

SECTION 2 Hazard Identification

2.1. Classification of the substance or mixture

Classification according to Regulation (EC) No. 1272/2008 [CLP]

| Flammable liquids, Category 3 | H226 | Flammable liquid and vapor. |
| --- | --- | --- |
| Skin corrosion/irritation, Category 2 | H315 | Causes skin irritation. |
| Serious eye damage/eye irritation, Category 1 | H318 | Causes serious eye damage. |
| Specific target organ toxicity – Single exposure, Category 3, repiratory syste. | H335 | May cause respiratory irritation. |
| Specific target organ toxicity – Single exposure, Category 3, Narcosis | H336 | May cause drowsiness or dizziness. |

Full text of H statements : see section 16

2.2. Label elements

Labelling according to Regulation (EC) No. 1272/2008 [CLP]

| Hazard pictograms (CLP) | : |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GHS02 | GHS07 | GHS05 |  |  |  |
| Signal word (CLP) | : | Danger |  |  |  |  |  |
| Hazard statements (CLP) | : | H226 – Flammable liquid and vapour H315 – Causes skin irritation. H317 - May cause an allergic skin reaction. H318 – Causes serious eye damage. H335 – May cause respiratory irritation. H336 - May cause drowsiness or dizziness |  |  |  |  |  |
| Precautionary statements (CLP) | : | P210 - Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P233 – Keep container tightly closed. P243 – Take action to prvent static discharges. P280 – Wear eye protection, protective gloves. P302 + P352 – IF ON SKIN: Wash with plenty of soap and water P332 + P313 – If skin irritation occurs: Get medical advice/attention P305 + P351+P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P310 – Immediately call a POISON CENTER or doctor/physician P370 + P378 - In case of fire: Use CO2, dry chemical, or foam to extinguish. P403 + P235 - Store in a well-ventilated place. Keep cool. P501 - Dispose of contents and container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation. |  |  |  |  |  |

2.3. Other Hazards

Contains no PBT and/or vPvB substances ≥ 0.1% assessed in accordance with REACH Annex XIII

The mixture does not contain substance(s) included in the list established in accordance with Article 59(1) of REACH for having endocrine disrupting properties, or substance(s) are not identified as having endocrine disrupting properties in accordance with the criteria set out in Commission Delegated Regulation (EU) 2017/2100 or Commission Regulation (EU) 2018/605 at a concentration equal to or greater than 0,1 %

SECTION 3 Composition/information on ingredients

3.1. Substances

Not applicable

3.2. Mixtures

| Name | Product identifier | % | Classification according Regulation (EC) No. 1272 [CLP] |
| --- | --- | --- | --- |
| Titanium Butoxide | CAS-No.: 5593-70-4 EC-No.: 227-006-8 | 97 | STOT SE 3, H336 STOT SE 3, H335 Skin Irrit. 2, H315 Flam. Liq. 3, H226 Eye Dam. 1, H318 |
| n-Butanol | CAS-No.: 71-36-3 EC-No.: 200-751-6 | ≤3 | STOT SE 3, H336 STOT SE 3, H335 Eye Dam. 1, H318 Skin Irrit. 2, H315 Acute Tox. 4 oral, H302 Flam. Liq. 3, H226 |
| Isopropyl titanate | CAS-No.: 546-68-9 EC-No.: 208-909-6 | <5 | Flam. Liq. 3, H226 Eye Irrit. 2, H319 STOT SE 3, H336 |

Full text of H- and EUH-phrases: see SECTION 16.

This mixture does not contain further substances fulfilling the criteria of hazard class "acute toxicity" according to CLP regulation.

This product does not contain substances of very high concern (Regulation (EC) No 1907/2006 (REACH), Article 57) in amounts above ≥0.1%.

SECTION 4 First aid measures

4.1. Description of necessary first-aid measures

| First-aid measures general | : | IF exposed or concerned: Get medical advice/attention. First aider: Pay attention to self-protection. Never give anything by mouth to an unconscious person. Give artificial respiration if necessary. Induce artificial respiration with mask fitted with one-way valve or other suitable device but, not mouth-to-mouth. |  |
| --- | --- | --- | --- |
| First-aid measures after inhalation | : | If breathing is difficult, remove victim to fresh air and keep at rest in a position comfortable for breathing. If the victim is unconscious: Lay in a stable manner on victim's side. Induce artificial respiration with mask fitted with one-way valve or other suitable device; not mouth-to-mouth. Call a physician immediately. |  |
| First-aid measures after skin contact | : | Remove affected clothing and wash all exposed skin areas with mild soap and water, followed by warm water rinse. If skin irritation or rash occurs: Get medical advice/attention. Wash contaminated clothing before reuse. |  |
| First-aid measures after eye contact | : | Immediately rinse with water for a prolonged period while holding the eyelids wide open. Remove contact lenses, if present and easy to do. Continue rinsing. Immediately transport the casualty to an eye doctor / hospital. Continue rinsing during the transport with isotonic saline solution, alternatively with water. |  |
| First-aid measures after ingestion | : | Rinse mouth and spit the fluids out. Do NOT induce vomiting. If vomiting occurs, the head should be kept low so that vomit does not enter the lungs. Call a poison center/doctor/physician if you feel unwell. |  |
| Self protection of the first-aider | : | First aid workers will be equipped with suitable personal protective equipment. |  |

4.2. Most important symptoms/effects, acute and delayed

Symptoms/effects after inhalation; :; Depression of the central nervous system, headaches, dizziness, drowsiness, loss of coordination. Inhalation of vapors may cause respiratory irritation.. Symptoms/effects after skin contact; :; Irritation (itching, redness, blistering). Repeated exposure may cause skin dryness or cracking.. Symptoms/effects after eye contact; :; Stinging, redness, itching, tears, blurred vision, swelling. Possible risk of irreversible effects.. Symptoms/effects after ingestion; :; Ingestion may cause nausea and vomiting. Gastrointestinal disturbances..

4.3. Indication of immediate medical attention and special treatment needed, if necessary

| Other medical advice or treatment | : | Further toxicology information in section 11 must be observed. |
| --- | --- | --- |

SECTION 5: Fire-fighting measures

5.1. Suitable (and unsuitable) extinguishing media

| Suitable extinguishing media | : | Water spray, extinguishing powder, alcohol-resistant foam, carbon dioxide (CO2). |
| --- | --- | --- |
| Unsuitable extinguishing media | : | Do not use waterjet |

5.2. Specific hazards arising from the chemical

| Fire hazard | : | Flammable liquid and vapour. |
| --- | --- | --- |
| Explosion hazard | : | Vapours are heavier than air and may travel considerable distance to an ignition source and flash back to source of vapors. |
| Hazardous decomposition products in case of fire | : | Toxic fumes may be released. Carbon dioxide. Carbon monoxide. Hydrocarbons. |

5.3. Special protective equipment and precautions for fire-fighters

| Firefighting instructions | : | In case of major fire and large quantities: Evacuate area. Fight fire remotely due to the risk of explosion. Fight fire with normal precautions from a reasonable distance. Do not enter fire area without proper protective equipment, including respiratory protection. Eliminate all ignition sources if safe to do so. |  |
| --- | --- | --- | --- |
| Protection during firefighting | : | Do not attempt to take action without suitable protective equipment. Self-contained breathing apparatus. Complete protective clothing. |  |

SECTION 6 Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

| General measures | : | Avoid all personal contact including breathing in the mist, spray, vapors. Do not take actions involving personal risks. Absorb spillage to prevent material-damage. Stop leak if safe to do so. Notify authorities if product enters sewers or public waters. |
| --- | --- | --- |

For non-emergency personnel

| Protective equipment | : | Wear recommended personal protective equipment. |
| --- | --- | --- |
| Emergency procedures | : | Ventilate spillage area. No open flames, no sparks, and no smoking. Avoid breathing dust/fume/gas/mist/vapours/spray. Avoid contact with skin and eyes. |

For emergency responders

| Protective equipment | : | Do not attempt to take action without suitable protective equipment. For further information refer to section 8: "Exposure controls/personal protection". |  |
| --- | --- | --- | --- |
| Emergency procedures | : | Evacuate unnecessary personnel. Ventilate spillage area. Stop leak if safe to do so. Prevent from entering sewers, basements and workpits, or any place where its accumulation can be dangerous. All equipment used when handling the product must be grounded. |  |
| Environmental precautions | : | Do not let the product reach soil, drains, sewers, or surface and ground water. Notify authorities if product enters sewers or public waters. |  |

6.2. Environmental Precautions

Avoid Release to the environment

6.3. Methods and materials for containment and cleaning up

| For containment | : | Absorb spilled material with sand or earth. Contain any spills with dikes or absorbents to prevent migration and entry into sewers or streams. Stop leak without risks if possible. |
| --- | --- | --- |
| Methods for cleaning up | : | Isolate all ignition sources. Stop leak if safe to do so. Contain and collect spillage with non-combustible, absorbent material e.g. sand, earth, vermiculite or diatomaceous earth and place in container for disposal according to local regulations (see Section 13). |
| Other Information. | : | Dispose of materials or solid residues at an authorized site. |
| For further information refer to section 8: "Exposure controls/personal protection",For further information refer to section 13 |  |  |

SECTION 7 Handling and storage

7.1. Precautions for safe handling

| Precautions for safe handling | : | Must be syphoned off in situ. Ensure adequate ventilation. Put on appropriate personal protective equipment (see Section 8). Keep away from sources of ignition and do not smoke. Take precautionary measures against electrostatic charging. Cool endangered containers with water. |
| --- | --- | --- |
| Hygiene measures | : | Immediately change contaminated clothing. Apply preventive skin protection. Wash hands and face after working with substance. For precautions see section 2.2. |
| Additional hazards when processed | : | Not expected to present a significant hazard under anticipated conditions of normal use. |

7.2. Conditions for safe storage, including incompatibilities

| Storage conditions | : | Keep in the original container in a cool well ventilated place. Keep container dry and tightly closed. Keep away from heat and sources of ignition. Storage class (TRGS 510): 3: Flammable liquids |
| --- | --- | --- |
| Incompatible materials | : | Do not store with oxidizing agents. |

7.3 Specifc end use(s)

| To be used as a catalyst for Vantablack 230 paint coating for aesthetic and engineering applications |  |
| --- | --- |

SECTION 8 Exposure controls/personal protection

8.1. Control parameters

| n-Butanol (71-36-3) |  |
| --- | --- |
| Occupational Exposure Limits |  |
| Local name | n-Butanol |
| WEL TWA (OEL STEL) IOELV TWA (OEL STEL) | 154 mg/m³ |
|  | 50 ppm |
| Regulatory reference | EH40/2005 (Fourth edition, 2020). HSE, Directive 2009/161/EU |

| Derived No-Effect Level (DNEL) - Titanium Butoxide: |  |
| --- | --- |
| Area of Use | Value |
| Worker; by inhalation; systemic (long term) | 127 mg/m³ |
| Consumer; by inhalation; systemic (long term) | 38 mg/m³ |
| Consumer; oral; systemic (long term) | 3.75 mg/kg bw/day |
| Consumer; dermal; systemic (long term) | 37.5 mg/kg bw/day |

| Predicted No Effect Concentration (PNEC) - Titanium Butoxide: |  |
| --- | --- |
| Area of Use | Value |
| Fresh water | 0.08 mg/ml |
| Marine water | 0.008 mg/ml |
| Sediment (freshwater) | 0.0687 mg/kg dry mass |
| Sediment (marine water) | 0.0069 mg/kg dry mass |
| Internittent release | 2.25 mg/l |
| Sewage treatment plant | 65 mg/l |
| Soil | 0.0168 mg/kg dry mass |

8.2. Appropriate engineering controls

| Appropriate engineering controls | : | Ensure good ventilation of the work station. |
| --- | --- | --- |
| Environmental exposure controls | : | Avoid release to the environment. Do not allow to enter drains or watercourses. |

8.3. Individual protection measures, such as personal protective equipment

Personal protective equipment:

Wear recommended personal protective equipment. Avoid all unnecessary exposure.

Hand protection:

Protective gloves are required at all times when handling the material, according to recognized standards such as EN374. Recommended glove types: Protective gloves made of fluorinated rubber: Thickness of the material: > 0,7 mm Breakthrough time: > 480 min Recommended glove types: Protective gloves made of butyl rubber: Thickness of the material: > 0,4 mm Breakthrough time: > 480 min Recommended glove types: Protective gloves made of nitrile rubber: Thickness of the material: > 0,4 mm Breakthrough time: > 480 min Please observe the instructions regarding permeability and breakthrough time which are provided by the glove supplier. Also take into consideration the specific local conditions under which the product is used, such as the danger of cuts, abrasion, and the contact time. Note that, due to the numerous external influences (such as temperature), a chemically resistant protective glove in daily use may have a service life that is considerably shorter than the measured break through time.

Eye protection:

Tight fitting protective goggles according to recognised standards such as EN166.

Skin and body protection:

Protective clothing

Respiratory protection:

If exposure by inhalation above the occupational exposure limit cannot be excluded, adequate respiratory protection equipment must be used. Suitable respiratory equipment: Respirator with a full face mask, according to acknowledged standards such as EN 136. Recommended Filter type: Gas filter type ABEK (certain inorganic, organic and acidic gases and vapours; ammonia/amines), according to acknowledged standards such as EN 14387. In case of mist, spray or aerosol exposure wear suitable personal respiratory protection and protective suit. Suitable respiratory equipment: Respirator with a full face mask, according to acknowledged standards such as EN 136. Recommended Filter type: Combined filter type ABEK-P2 (certain inorganic, organic and acidic gases and vapors; ammonia/amines; particles), according to acknowledged standards such as EN 14387. For long or intense exposure, use respiratory protective equipment. Suitable respiratory equipment: Positive pressure self contained breathing apparatus, according to acknowledged standards such as EN 137. Observe the equipment manufacturer's information and wear time limits for respirators.

Personal protective equipment symbol(s):

Other information:

Do not eat, drink or smoke during use.

SECTION 9 Physical and chemical properties

9.1. Basic physical and chemical properties

| Physical state | : | Liquid |
| --- | --- | --- |
| Appearance | : | High viscosity liquid |
| Color | : | Light yellow |
| Odor | : | Alcoholic |
| pH | : | Not applicable, reacts with water |
| Melting point | : | -50°C |
| Freezing point | : | -70°C |
| Boiling point | : | 312°C at 1.013 hPa |
| Flash point | : | 40 °C Abel method Closed cup (ISO13736) |
| Flammability (solid, gas) | : | Lower: 2% (Vol-%) Upper: 12% (Vol-%) |
| Vapor pressure | : | 0.02 kPa at 21°C |
| Relative vapor density at 20°C | : | No data available |
| Relative density | : | 0.991-1.001 g/ml (20°C) |
| Solubility | : | No data available |
| Partition coefficient n-octanol/water (Log Pow) | : | No data available |
| Auto-ignition temperature | : | No data available |
| Decomposition temperature | : | 300-319°C (thermal) |
| Viscosity, kinematic | : | 60 mPa.s (dynamic, 25°C) |
| Explosion limits | : | Lower explosion limit: 2 vol % Upper explosion limit: 13 vol % |
| Particle characteristics | : | Molecular weight 340.36 |

9.2. Other Information

| No additional information available. |  |  |
| --- | --- | --- |

SECTION 10 Stability and reactivity

10.1. Reactivity

Vapour/air-mixtures are explosive at intense warming.

10.2. Chemical stability

Stable under normal conditions of use.

10.3. Possibility of hazardous reactions

No data available.

10.4. Conditions to avoid

Heating.

10.5. Incompatible materials

Reacts with water. The reaction takes place with the formation of harmful substances.

10.6. Hazardous decomposition products

n-Butanol

SECTION 11 Toxicological information

11.1. Information on toxicological effects

| Acute toxicity (oral) | : | Based on the available data acute toxic effects are not expected after single oral exposure. |
| --- | --- | --- |
| Acute toxicity (dermal) | : | Not classified |
| Acute toxicity (inhalation) | : | No data available |

| Route of exposure | Result/Effect | Species/Test system | Source |
| --- | --- | --- | --- |
| Oral | LD50: 4640 mg/kg | Ray | Literature |

| Skin corrosion/irritation | : | After contact to the skin irritation of the skin are to be expected. |
| --- | --- | --- |

| Respiratory or skin sensitization | : | For this endpoint no toxicological test data is available for the whole product. |
| --- | --- | --- |

| Carcinogenicity | : | For this endpoint no toxicological test data is available for the whole product. |
| --- | --- | --- |

| Reproductive toxicity | : | For this endpoint no toxicological test data is available for the whole product. |
| --- | --- | --- |

| STOT-repeated exposure | : | For this endpoint no toxicological test data is available for the whole product. |
| --- | --- | --- |

| Aspiration hazard | : | No data known |
| --- | --- | --- |

SECTION 12 Ecological information

12.1. Ecotoxicity

| Ecology - general | : | There are no data available on the mixture itself. Do not allow to enter water drains or courses. |
| --- | --- | --- |
| Hazardous to the aquatic environment, short–term (acute) | : | Not classified |
| Hazardous to the aquatic environment, long–term (chronic) | : | Not classified |

12.2. Persistence and degradability

Hydrolytically instable, reacts with water.

12.3. Bioaccumulative potential

Bioaccumulation is not expected to occur.

12.4. Mobility in soil

No additional information available

12.5. Results of PBT and vPvB assessment

| No additional information available |  |  |
| --- | --- | --- |

## 12.6. Endocrine disrupting properties

| No additional information available |  |  |
| --- | --- | --- |

## 12.7. Other adverse effects

| No negative effects known |  |  |
| --- | --- | --- |

SECTION 13 Disposal considerations

| Regional waste regulation | : | Disposal must be done according to official regulations. |
| --- | --- | --- |
| Waste treatment methods | : | Dispose of contents/container in accordance with licensed collector’s sorting instructions. |
| Sewage disposal recommendations | : | Disposal must be done according to official regulations. |
| Product/Packaging disposal recommendations | : | Disposal must be done according to official regulations. |
| Additional information | : | Do not re-use empty containers. |
| Ecological waste information | : | Avoid release to the environment. |

SECTION 14 Transport information

In accordance with ADR / ADN / IMDG / IATA

|  | Land Transport (ADR/RID) | Inland waterway transport (ADN) | Sea Transport (IMDG) | Air transport (ICAO-TI/IATA-DGR) |
| --- | --- | --- | --- | --- |
| 14.1. UN number | 1993 | 1993 | 1993 | 1993 |
| 14.2. Proper Shipping Name | titanium tetrabutanolate and n-butanol | titanium tetrabutanolate and n-butanol | titanium tetrabutanolate and n-butanol | titanium tetrabutanolate and n-butanol |
| 14.3. Transport hazard class(es) | 3 | 3 | 3 | 3 |
| Hazard Labels | Flam. liq. | Flam. liq. | Flam. liq. | Flam. liq. |
| 14.4. Packing group | 3 | 3 | 3 | 3 |
| 14.5. Environmental hazards | unknown | unknown | unknown | unknown |

14.6. Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code

Bulk transport in tankers is not intended

14.7. Special precautions for user

Relevant information in other sections has to be considered. Always transport in closed containers that are upright and secure. Ensure that persons transporting the product know what to do in the event of an accident or spillage.

SECTION 15 Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

| EU Regulations |  |  |
| --- | --- | --- |
| REACH Annex XVII (Restriction List) | : | Contains no substance(s) listed on REACH Annex XVII (Restriction Conditions) |
| REACH Annex XIV (Authorisation List) | : | Contains no substance(s) listed on REACH Annex XIV (Authorisation List) |
| REACH Candidate List (SVHC) | : | Contains no substance(s) listed on the REACH Candidate List |
| PIC Regulation (Prior Informed Consent) | : | Contains no substance(s) listed on the PIC list (Regulation EU 649/2012 concerning the export and import of hazardous chemicals) |
| POP Regulation (Persistent Organic Pollutants) | : | Contains no substance(s) listed on the POP list (Regulation EU 2019/1021 on persistent organic pollutants) |
| Ozone Regulation (2024/590) | : | Contains no substance(s) listed on the Ozone Depletion list (Regulation EU 2024/590 on substances that deplete the ozone layer) |
| Council Regulation (EC) for the control of dual-use items | : | Contains no substance subject to the COUNCIL REGULATION (EC) for the control of dual-use items |

Explosives Precursors Regulation (EU 2019/1148) Does not contain substance(s) listed on the Explosives Precursors list (Regulation EU 2019/1148 on the marketing and use of explosives precursors) ANNEX II REPORTABLE EXPLOSIVES PRECURSORS List of substances on their own or in mixtures or in substances for which suspicious transactions and significant disappearances and thefts are to be reported within 24 hours.

Drug Precursors Regulation (EC 273/2004) Does not contain substance(s) listed on the Drug Precursors list (Regulation EC 273/2004 on the manufacture and the placing on market of certain substances used in the illicit manufacture of narcotic drugs and psychotropic substances)

15.2. EU-Regulations

EU-Regulations

EU regulations: Not a hazardous substance according to the summation method of Regulation (EC) 1272/2008 (CLP), its various amendments and adaptations and Directive 67/548/EEC.

Authorisations and/or restrictions on use

Authorisations : Not applicable

Restrictions on use : Not applicable

Other EU regulations:

SI 2002/1689: CHIP Regulations 2002

SI 2002/2677: COSHH Regulations 2002

SI 1999/3242: Management of Health & Safety at Work Regulations 1999

Health & Safety at Work Act 1974

SI 1993/1643: Environmental Protection Act 1993 & Subsidiary Regulations.

Other national and local measures relating to the workplace, pollution control, environmental protection and waste control.None

15.3. Chemical Safety Assessment

For this mixture a chemical safety assessment is not required.

SECTION 16 Other information

| 16.1. Full text of hazard classes and H-statements |  |
| --- | --- |
| H226 | Flammable liquids Category 3; Flammable liquid and vapour. |
| H302 | Acute toxicity Category 4; Harmful if swallowed. |
| H315 | Causes skin irritation |
| H319 | Causes serious eye irritation |
| H318 | Serious eye damage/eye irritation Category 1; Causes serious eye damage. |
| H226 | Flammable liquids Category 3; Flammable liquid and vapour. |
| H335 | Specific target organ toxicity - single exposure Category 3; May cause respiratory irritation. |
| H336 | May cause drowsiness or dizziness |

| 16.2. Full text of precautionary statements and P-statements |  |
| --- | --- |
| P210 | Keep away from heat, hot surfaces, sparks,open flames and other ignition sources. No smoking |
| P233 | Keep container tightly closed |
| P243 | Take action to prevent static discharges |
| P180 | Wear eye protection, protective gloves |
| P302 + P352 | IF ON SKIN: Wash with plenty of soap and water |
| P332 + P313 | If skin irritation occurs: Get medical advice/attention |
| P305 + P351 + P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing |
| P310 | Immediately call a POSION CENTRE or doctor/physician |
| P370 + P378 | In case of fire: Use CO2, dry chemical, or foam to extinguish |
| P403 + P235 | Store in a well-ventiliated place. Keep cool. |
| P501 | Dispose of content and container to hazardous or special waste collection point, in accordance with local, regional. National and/or international regulation. |

| 16.3. Abbreviations and acronyms |  |
| --- | --- |
| ACGIH | American Conference of Government Industrial Hygienists |
| ADN | European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways |
| ADR | European Agreement concerning the International Carriage of Dangerous Goods by Road |
| ATE | Acute Toxicity Estimate |
| BCF | Bioconcentration factor |
| BLV | Biological limit value |
| BOD | Biochemical oxygen demand (BOD) |
| CAS-No. | Chemical Abstract Service number |
| CLP | Classification Labelling Packaging Regulation; Regulation (EC) No 1272/2008 |
| COD | Chemical oxygen demand (COD) |
| CSA | Chemical safety assessment |
| DMEL | Derived Minimal Effect level |
| DNEL | Derived-No Effect Level |
| EC-No. | European Community number |
| EC50 | Median effective concentration |
| ED | Endocrine disruptor |
| EN | European Standard |
| EWC | European waste catalogue |
| IARC | International Agency for Research on Cancer |
| IATA | International Air Transport Association |
| IMDG | International Maritime Dangerous Goods |
| LC50 | Median lethal concentration |
| LD50 | Median lethal dose |
| LOAEL | Lowest Observed Adverse Effect Level |
| Log Kow | Partition coefficient n-octanol/water (Log Kow) |
| Log Pow | Partition coefficient n-octanol/water (Log Pow) |
| MAK | maximum workplace concentration |
| NOAEC | No-Observed Adverse Effect Concentration |
| NOAEL | No-Observed Adverse Effect Level |
| NOEC | No-Observed Effect Concentration |
| N.O.S. | Not Otherwise Specified |
| OECD | Organization for Economic Co-operation and Development |
| OEL | Occupational Exposure Limit |
| OSHA | Occupational Safety Health Administration |
| PBT | Persistent Bioaccumulative Toxic |
| PNEC | Predicted No-Effect Concentration |
| PPE | Personal protection equipment |
| RID | Regulations concerning the International Carriage of Dangerous Goods by Rail |
| SDS | Safety Data Sheet |
| STP | Sewage treatment plant |
| TF | Technical function |
| ThOD | Theoretical oxygen demand (ThOD) |
| TLM | Median Tolerance Limit |
| TWA | Time Weighted Average |
| VOC | Volatile Organic Compounds |
| vPvB | Very Persistent and Very Bioaccumulative |
| UFI | Unique Formula Identifier |

## 16.4. Training Advice

Hands on training to be administered by authorised representatives of Surrey Nanosystems Ltd only. Use of the material without training is done so at operator’s risk.

## 16.5. Further Information

Further information: IMPORTANT NOTE The information in this data sheet is not intended to be exhaustive and is based on the present state of our knowledge and on current laws: any person using the product for any purpose other than that specifically recommended in the technical data sheet without first obtaining written confirmation from us as to the suitability of the product for the intended purpose does so at his own risk. It is always the responsibility of the user to take all necessary steps to fulfil the demands set out in the local rules and legislation. Always read the Material Data Sheet and the Technical Data Sheet for this product if available. All advice we give or any statement made about the product by us (whether in this data sheet or otherwise) is correct to the best of our knowledge but we have no control over the quality or the condition of the substrate or the many factors affecting the use and application of the product. Therefore, unless we specifically agree in writing otherwise, we do not accept any liability whatsoever for the performance of the product or for any loss or damage arising out of the use of the product. All products supplied and technical advice given are subject to our standard terms and conditions of sale. You should request a copy of this document and review it carefully. The information contained in this data sheet is subject to modification from time to time in the light of experience and our policy of continuous development. It is the user's responsibility to verify that this data sheet is current prior to using the product. Brand names mentioned in this data sheet are trademarks of or are licensed to Surrey Nanosystems Ltd